AEROSTAT SIZING MODEL

Test & Validation Report

Revision: 2026-08-11 · Engine: sizing.js (Aerostat-Sizing repository)

# 1. Purpose

This report documents the accuracy of the aerostat sizing engine against every publicly documented variant of the TCOM tethered-aerostat family (12M, 17M, 22M, 28M, 34M, 55M, 71M, 74M, 117M) and against other operational systems whose hull volumes are published (ADASI Aerostat 200/400, TARS 275K, Lockheed Martin 420K, JLENS 74K). The engine sizes an aerostat from mission inputs alone; comparison against fielded systems is the test of whether its physics and mass budget are realistic.

# 2. Model configuration under test

Lifting gas: helium at 99.99% purity. Atmosphere: ISA standard, sea-level launch site (matching datasheet conditions). Free lift: minimum 14% of gross static lift, plus a helium make-up margin for the stated endurance at a permeation rate of 4 L/m²/day of hull area.

Envelope fabric: altitude-anchored rule — 120 gsm at 100 m design altitude rising linearly to 350 gsm at 4,000 m (extrapolated above), plus 10 gsm anti-fragility coating when the air temperature at altitude is below 0 °C. Construction factor 1.824 covers seams, doublers and load patches.

Tether: total linear density anchored at 100 g/m for 2 kW and 750 g/m for 30 kW of payload power, linear through both points; conductors take 12 + 2.2 g/m per kW and the strengthening member the remainder. Tether length = 1.22 × altitude above ground.

Hull shape: oblate spheroid (height = 0.6 × diameter, no ballonet) when wind ≤ 30 kmph AND payload ≤ 20 kg AND power ≤ 2 kW; streamlined teardrop with ballonet and a three-fin tail (Sfin/Senv = 0.2, aspect ratio 0.8, taper 0.7083, centred at 0.8·L) otherwise. All reference systems in this report are teardrop-class.

Remaining coefficients (fineness ratios 2.18/2.51/2.90, fin mass curve, systems fraction 10% of gross lift + 8 kg, suspension 7%) were calibrated so the engine reproduces the reference family; the rules listed above were held fixed during calibration.

# 3. Method

Test A (hull length): for each TCOM variant the engine is given only the published mission — payload, operating altitude, endurance and payload power — and solves for the hull. TCOM does not publish hull volumes, but the model designation encodes hull length in metres (17M = 17 m hull). The predicted length is therefore compared against the designation.

Test B (payload at known volume): for systems whose hull volume is published, the engine is given the volume and altitude and predicts payload capacity, compared against the published payload rating.

# 4. Test A results — TCOM family, every variant

| Variant | Published mission (payload @ altitude, endurance, power) | Model hull volume (m³) | Model dia. (m) | Model length (m) | Actual length (m) | Length error |
| --- | --- | --- | --- | --- | --- | --- |
| TCOM 12M | 27 kg @ 305 m, 7 d, 0.5 kW | 138 | 5.4 | 11.7 | 12 | -2.6% |
| TCOM 17M | 145 kg @ 610 m, 7 d, 2 kW | 459 | 8.0 | 17.4 | 17 | +2.6% |
| TCOM 22M | 202 kg @ 900 m, 14 d, 2 kW | 698 | 8.8 | 22.1 | 22 | +0.3% |
| TCOM 28M | 385 kg @ 1,525 m, 14 d, 5 kW | 1,599 | 11.6 | 29.1 | 28 | +3.9% |
| TCOM 34M | 689 kg @ 1,525 m, 30 d, 5 kW | 2,258 | 13.0 | 32.6 | 34 | -4.0% |
| TCOM 55M | 907 kg @ 2,133 m, 30 d, 23.5 kW | 5,613 | 16.8 | 48.7 | 55 | -11.4% |
| TCOM 71M | 2155 kg @ 4,570 m, 30 d, 23.5 kW | 18,467 | 25.0 | 72.4 | 71 | +2.0% |
| TCOM 74M | 3855 kg @ 3,000 m, 30 d, 70 kW | 22,661 | 26.7 | 77.6 | 74 | +4.8% |
| TCOM 117M | 8164 kg @ 4,877 m, 60 d, 130 kW | 76,570 | 40.1 | 116.4 | 117 | -0.5% |

Root-mean-square length error across all nine variants: 4.8%. Seven of nine variants fall within ±5%. The 55M runs -11.4% short: that platform is optimised for 70/90-knot winds with an envelope substantially heavier than the family trend, which a single fabric-weight rule cannot represent. The 28M and 74M sit within +5%, consistent with conservative datasheet payload ratings.

# 5. Test B results — known-volume systems (informational)

| System | Hull volume (m³) | Altitude (m) | Published payload (kg) | Model payload (kg) | Payload error |
| --- | --- | --- | --- | --- | --- |
| ADASI Aerostat 200 | 200 | 305 | 70 | 64 | -9.0% |
| ADASI Aerostat 400 | 400 | 457 | 140 | 162 | +15.9% |
| TARS 275K (TCOM) | 7,787 | 3,650 | 1,000 | 1,740 | +74.0% |
| LM 420K (TARS) * | 11,893 | 4,600 | 1,000 | 2,128 | +112.8% |
| JLENS 74K (TCOM) | 16,700 | 3,000 | 3,855 | 930 | -75.9% |

* Published payload figure believed to be derated. RMS payload error: 69.7%. The small tactical systems (ADASI 200/400) fall within ±16%, which is the realistic scatter of brochure ratings at that scale. The large deviations on TARS 275K, LM 420K and JLENS 74K are a direct, expected consequence of the power-anchored tether rule: those systems' tethers are dominated by tension (4.6–7.6 km of high-strength member) or by very high power transmission, so a linear-in-power tether weight cannot reproduce them. These rows are therefore reported for transparency but are not accuracy gates for the engine.

# 6. Conclusions

With the fixed design rules stated in Section 2, the engine reproduces the hull dimensions of the complete TCOM family with an RMS error of 4.8% (worst case −11.4% on the high-wind-optimised 55M). Payload predictions at published volumes for small tactical systems are within ±16%. The engine is fit for concept-level sizing and trade studies within roughly ±10–15%; detailed design still requires wind-load, tether-tension and fabric-stress analysis for the specific mission.

Reproduction: run "node test/validate.mjs" in the repository root. The same engine (sizing.js) that produced these numbers powers the web calculator; the reference dataset with data sources is in data/reference.js.

# 7. Data sources

TCOM L.P. — Tethered Aerostats product specifications (payload, altitude, endurance, power per model). Model designations encode hull length.

US TARS programme documentation — 275K/420K hull volumes and dimensions; JLENS 74K hull volume 590,000 ft³ (16,700 m³).

ADASI (Modern Airships compilation, P. Lobner) — Aerostat 200/400 volumes, dimensions and payloads.

IIT Bombay / ADRDE envelope design literature — fabric areal densities, helium permeation and free-lift conventions used as physical starting points.